SCHOOL OF ARTIFICIAL INTELLIGENCE | COIMBATORE

PROJECT REPORT

WhisperComm : AN END-TO-END AM COMMUNICATION SYSTEM WITH AI-ENHANCED SPEECH RECOVERY

Project Lead
Mr. NIRMAL RAMAMOORTHY
B. Tech in Artificial Intelligence and Data Science
Amrita School of Artificial Intelligence
Amrita School of Engineering, Coimbatore
Batch of 2028

Contributors
N. SANJANA REDDY

Done by

Supervised by

DR. JYOTHISH LAL. G

Assistant Professor (Sr.Gr.), School of Artificial Intelligence

Amrita Vishwa Vidyapeetham, Coimbatore

Abstract—Amplitude Modulation (AM) is a foundational communication technology persisting in critical sectors like aviation and broadcasting. However, its inherent vulnerability to additive noise significantly impairs signal intelligibility, posing a substantial barrier for automated monitoring and transcription systems. This report details a rigorous, end-to-end simulation pipeline created to investigate and quantify the efficacy of deep learning-based speech enhancement for rehabilitating demodulated AM signals.

This report presents WhisperComm, a complete end-to-end simulated radio communication system that integrates classical Digital Signal Processing (DSP) with state-of-the-art Artificial Intelligence to achieve robust, high-accuracy speech recovery under severe channel noise conditions.

The system transmits high-quality speech from the LibriSpeech corpus over a simulated Amplitude Modulation (AM) channel corrupted by pink noise at five distinct Signal-to-Noise Ratio (SNR) levels: 25, 18, 12, 8, and 5 dB. On the receiver side, a Synchronous (Coherent) Demodulator recovers the baseband audio. An AI denoising model (Facebook DNS-64) is then applied to suppress residual channel noise, and OpenAI's Whisper large-v3 model performs Automatic Speech Recognition (ASR) to produce the final transcription. Performance is quantified using the Word Error Rate (WER) metric.

A key engineering discovery of this work is that applying AI denoising unconditionally is detrimental — at high SNR levels the denoiser damages the signal by introducing spectral artefacts that degrade ASR accuracy. This led to the design and implementation of a Dynamic SNR Routing Pipeline, which activates the AI denoiser only when the channel degrades below a critical threshold (5 dB), achieving the minimum possible WER across all channel conditions. The Smart Pipeline reduced the average system WER by approximately 18 percentage points compared to a naively applied AI-only approach.

This study demonstrates that the highest-performing AI-DSP hybrid systems are those grounded in rigorous classical signal processing principles, and that context-aware AI deployment is essential in telecommunications applications.

Keywords—Speech Enhancement, Deep Learning, Amplitude Modulation (AM), Automatic Speech Recognition (ASR), Denoising, Word Error Rate (WER), Signal-to-Noise Ratio (SNR), Computational Audio, Signal Restoration, Pink Noise.

Acknowledgment

We would like to express our deepest gratitude to our project guide, Dr. Jyothish Lal. G Assistant Professor (Sr.Gr.), for his invaluable guidance, unwavering support, and insightful feedback throughout the duration of this project. Their expertise was instrumental in shaping our research and navigating the technical challenges we encountered.

Our gratitude also goes to our colleagues and friends who provided their encouragement and engaged in fruitful discussions that helped refine our ideas.

Finally, we acknowledge the creators and maintainers of the open-source software and datasets that made this work possible, including the developers of Python, PyTorch, the Hugging Face libraries, and the curators of the LibriSpeech corpus.

II. INTRODUCTION

Modern digital communication systems face an ever-present challenge: the reliable transmission and recovery of intelligible speech over imperfect, noise-corrupted channels. While classical signal processing has long provided the mathematical foundation for radio communication, recent advances in deep learning have introduced powerful new tools for speech restoration. The central question motivating this project is whether, and under what conditions, AI-based processing genuinely improves the end-to-end performance of a communication chain compared to classical DSP alone.

WhisperComm addresses this question by constructing a complete, instrumented simulation environment encompassing every stage of a real-world radio link: source audio, transmitter modulation, channel impairment, receiver demodulation, AI enhancement, and automatic speech recognition. By measuring the Word Error Rate (WER) at the output of the chain under five channel conditions, the study produces quantitative evidence regarding the contribution — and the limitations — of deep learning within a classical DSP framework.

The principal objectives of this project are:

- Design and implement an AM transceiver simulation operating at a 16 kHz sample rate.

- Model realistic channel noise using a pink noise (1/f) process across a range of SNR levels.

- Evaluate Synchronous (Coherent) Demodulation as an upgrade over Envelope Detection.

- Integrate Facebook DNS-64 for AI-based noise suppression and quantify its effect on ASR accuracy.

- Discover the conditions under which AI enhancement helps or harms downstream recognition.

- Design a Dynamic SNR Routing system that achieves optimal WER across all channel conditions

The report is structured as follows. Section III reviews the relevant literature on AM modulation, speech enhancement, and ASR evaluation. Section IV presents the full system design and methodology. Section V details the implementation. Sections VI and VII present results and discussion respectively. Section VIII concludes the report and proposes directions for future work.

III. LITERATURE REVIEW / BACKGROUND STUDY

A. Amplitude Modulation Principles and Limitations

Amplitude Modulation is one of the oldest and most extensively studied forms of analogue radio transmission. In conventional AM, a message signal m(t) modulates the amplitude of a sinusoidal carrier c(t) = A cos(2π fc t) to produce the transmitted signal s(t) = [1 + m(t)] × c(t). The factor of 1 ensures the envelope remains non-negative for signals with |m(t)| ≤ 1, allowing simple envelope detection at the receiver (Haykin, 2001).

A fundamental constraint of AM over band-limited digital systems is the aliasing problem. The Nyquist-Shannon sampling theorem states that a discrete signal sampled at fs can represent frequencies up to fs/2 (the Nyquist frequency). When the baseband signal occupies frequencies up to fmax and is modulated onto a carrier fc, the upper sideband extends to fc + fmax. If this exceeds the Nyquist frequency, irreversible spectral overlap (aliasing) corrupts the signal. For the 16 kHz sample rate and 4 kHz carrier used in this project, a pre-modulation Low-Pass Filter (LPF) with a 3.8 kHz cutoff was required to prevent aliasing, constraining the voice bandwidth to [0, 3.8 kHz] and the transmitted spectrum to [200 Hz, 7.8 kHz], safely within the 8 kHz Nyquist boundary (Proakis & Manolakis, 2007).

Coherent (Synchronous) Demodulation, in which the receiver multiplies the incoming signal by a locally generated replica of the carrier and low-pass filters the result, produces significantly lower distortion than Envelope Detection for band-limited systems. Envelope Detection performs well only when the carrier frequency greatly exceeds the message bandwidth; when the ratio fc / fmax is small (as in this project, where it is approximately 1.05), envelope detection introduces severe nonlinear distortion that propagates through all downstream processing stages (Lathi & Ding, 2009).

B. Classical and Modern Speech Enhancement

Speech enhancement refers to the class of algorithms that seek to improve the perceptual quality and intelligibility of speech corrupted by additive noise, reverberation, or other distortions. Classical methods include spectral subtraction (Boll, 1979), Wiener filtering (Lim & Oppenheim, 1979), and minimum mean-square error (MMSE) spectral estimators (Ephraim & Malah, 1984). These techniques rely on statistical models of the noise process and are computationally efficient, but they introduce 'musical noise' artefacts when the noise is non-stationary.

Deep learning approaches have fundamentally changed the speech enhancement landscape. Early neural approaches such as Deep Denoising Autoencoders (Vincent et al., 2008) were followed by fully-supervised deep neural networks trained on matched pairs of clean and noisy speech. Facebook's DEMUCS architecture, underpinning the DNS-64 model used in this project, is a convolutional encoder-decoder network trained on the DNS Challenge dataset (Defossez et al., 2020). It operates directly in the time domain at 16 kHz, eliminating the need for Short-Time Fourier Transform (STFT) spectral estimation.

A critical and under-discussed limitation of data-driven speech enhancement models is domain mismatch. DNS-64 was trained on environmental acoustic noise (background noise in rooms, traffic, appliances) and not on AM radio channel noise or coherent demodulation artefacts. When presented with out-of-distribution noise types, deep models can aggressively suppress speech-carrying frequencies, a phenomenon that worsens recognition accuracy rather than improving it (Kinoshita et al., 2020). This project provides a direct empirical demonstration of this effect.

C. ASR as an Evaluation Metric

Automatic Speech Recognition systems have matured to the point where they serve as objective, reproducible evaluation instruments for speech quality. The Word Error Rate (WER) — defined as (Substitutions + Deletions + Insertions) / Reference Word Count — captures not just acoustic intelligibility but the full linguistic fidelity of a recovered signal. It is the standard metric used in both the CHiME and DNS speech enhancement challenges (Barker et al., 2015).

OpenAI Whisper (Radford et al., 2022) represents the current state of the art in general-purpose ASR. The large-v3 variant was trained on 680,000 hours of weakly supervised multilingual data and achieves near-human performance on LibriSpeech test-clean. Crucially, Whisper is internally robust to moderate noise — its training data includes noisy recordings — making it an extremely demanding downstream evaluator. A speech enhancement system must produce genuinely higher-quality audio to show improved WER; it cannot rely on Whisper's built-in noise robustness to mask processing artefacts.

The LibriSpeech corpus (Panayotov et al., 2015), derived from LibriVox audiobook recordings, provides reference-quality speech at 16 kHz with professionally transcribed ground truth. Its test-clean split is universally used as the benchmark for ASR research, making it an ideal source for the ground truth signal in this project.

IV. SYSTEM DESIGN & METHODOLOGY

The WhisperComm system is organised as a sequential pipeline with clearly defined processing stages. Figure 1 (see architecture diagram) illustrates the full signal flow from the clean speech source to the final transcription output. Each stage is described in detail below.

A. Dataset and Ground Truth Signal

The source material for all experiments is a single utterance drawn from the LibriSpeech test-clean split, specifically sample ID 121-121726-0000. This sample is 16-bit PCM audio sampled at 16,000 Hz and reads: "ALSO A POPULAR CONTRIVANCE WHEREBY LOVE MAKING MAY BE SUSPENDED BUT NOT STOPPED DURING THE PICNIC SEASON." The choice of a single carefully selected utterance, rather than a large batch evaluation, allows for precise spectral and temporal analysis at each processing stage.

The ground truth transcript was normalised for WER computation by converting to lowercase and stripping punctuation, producing the reference string: "also a popular contrivance whereby love making may be suspended but not stopped during the picnic season" (17 words). All WER values in this report are computed against this normalised reference.

B. Amplitude Modulation Simulation

The transmitter implements Double-Sideband Full-Carrier (DSB-FC) AM modulation. Prior to modulation, a sixth-order Butterworth LPF with a cutoff frequency of 3,800 Hz is applied to the source audio. This pre-filter is essential: with a carrier frequency of fc = 4,000 Hz and a Nyquist limit of 8,000 Hz, any voice energy above 4,000 Hz would alias after modulation. The filter attenuates these components by more than 60 dB.

After filtering, the message signal m(t) is normalised to the range [−1, +1] and the AM signal is formed as:

s(t) = [1 + m(t)] × cos(2π × 4000 × t)

The modulation index, defined as the peak absolute value of m(t), is equal to 1.000 by construction, corresponding to 100% AM modulation with full carrier. The resulting transmitted signal has an expected double-sided spectrum centred at ±4,000 Hz with sidebands spanning 200 Hz to 7,800 Hz.

C. Noisy Channel Modeling

The channel model adds coloured noise to the transmitted AM signal to simulate realistic electromagnetic interference. Pink noise (also called 1/f noise or flicker noise) was selected because it closely approximates the spectral character of natural electronic interference in the audio band; its power spectral density (PSD) decreases at 10 dB per decade, matching observed interference in AM broadcast receivers.

Pink noise is generated computationally via FFT spectral shaping: a white noise sequence is transformed to the frequency domain, each frequency bin is scaled by 1/√f, and the result is inverse-transformed. The DC bin is treated separately to avoid division by zero. The noise sequence is normalised to unit variance before scaling.

The received noisy signal is constructed as:

r(t) = s(t) + η(t) × √(P_s / 10^(SNR/10))

where P_s is the mean power of s(t), η(t) is the unit-variance pink noise, and SNR ∈ {25, 18, 12, 8, 5} dB. This formulation guarantees that the measured signal-to-noise power ratio at the receiver input precisely matches the target SNR.

D. Demodulation: Envelope Detection vs. Synchronous Demodulation

The initial implementation of the receiver used Envelope Detection, which computes the instantaneous envelope of r(t) as |r(t)| and applies a Butterworth LPF to recover the baseband signal. This approach is suitable when the carrier frequency greatly exceeds the message bandwidth. However, in this system the ratio fc / fmax ≈ 1.05, causing the rectification operation to introduce severe harmonic distortion that degraded all downstream processing.

The receiver was upgraded to Synchronous (Coherent) Demodulation. The incoming signal r(t) is multiplied by a locally generated carrier cos(2π fc t) that is perfectly phase-coherent with the transmitter carrier:

v(t) = r(t) × cos(2π fc t)

Multiplication shifts the baseband components to DC and the AM sideband artefacts to 2fc = 8,000 Hz. A sixth-order Butterworth LPF (cutoff 3,800 Hz) then removes all high-frequency components, leaving a clean baseband estimate. The DC offset introduced by the carrier term is removed by subtracting the mean, and the signal is normalised to [−1, +1]. This upgrade reduced the baseline WER at 12 dB from 70.59% to 17.65%, a dramatic improvement demonstrating the importance of correct receiver design.

E. AI-Based Speech Enhancement

Facebook's DNS-64 model, accessed through the denoiser Python package, is applied to the demodulated baseband audio before ASR transcription. DNS-64 is a DEMUCS-based convolutional encoder-decoder that processes raw audio waveforms. It accepts input tensors of shape [batch, channels, time] and returns denoised audio of the same shape.

Implementation required careful attention to tensor dimensionality. The demodulated signal is a 1-D NumPy array of shape (T,). To satisfy the model's expected input format, the array must be promoted to shape (1, 1, T) by two successive unsqueeze operations before inference. After inference, the output tensor is reduced back to a 1-D array by a global squeeze operation. The model is executed in torch.no_grad() context to suppress gradient computation and reduce VRAM consumption.

F. Evaluation Framework

Speech recognition is performed using OpenAI Whisper (whisper-small for development, scalable to large-v3 for final evaluation) via the HuggingFace Transformers library. To avoid instabilities in the pipeline abstraction associated with the 'num_frames' KeyError bug in certain library versions, the system uses direct WhisperProcessor / WhisperForConditionalGeneration inference with explicit tensor management.

Each audio signal (baseline demodulated and AI-enhanced) is passed through the processor to generate log-Mel spectrogram features, which are forwarded through the model with beam search (num_beams = 5) and forced English language decoding. The WER is computed using the HuggingFace evaluate library's jiwer-backed WER metric, with both hypothesis and reference normalised to lowercase stripped text.

V. IMPLEMENTATION

A. Environment and Tools

All experiments were conducted within a Google Colab notebook environment, configured with an NVIDIA T4 GPU (16 GB VRAM) and the Python 3.12 runtime. The GPU was essential for loading DNS-64 and Whisper within the VRAM budget of a single notebook session. Package installation was performed programmatically using subprocess.run with sys.executable to guarantee installation into the active Python environment.

B. Core Libraries and Models

The following libraries and models were used throughout the implementation:

| Library / Model | Version | Purpose |
| --- | --- | --- |
| PyTorch | 2.x (CUDA) | Tensor operations, GPU inference |
| HuggingFace Transformers | 4.x | Whisper ASR pipeline & manual inference |
| HuggingFace datasets | 2.x | LibriSpeech data loading |
| HuggingFace evaluate | 0.x | WER metric computation (jiwer backend) |
| denoiser (Facebook) | 0.1.5 | DNS-64 pretrained denoising model |
| SciPy signal | 1.x | Butterworth filter design & filtfilt |
| librosa | 0.10.x | Spectrograms, STFT, amplitude_to_db |
| NumPy | 1.26.x | Array operations, FFT pink noise generation |
| Matplotlib + GridSpec | 3.8.x | All visualisations and dashboards |
| OpenAI Whisper (HF) | small / large-v3 | End-to-end ASR evaluation |

C. Key Simulation Parameters

The following parameters govern all experiments in this study:

| Parameter | Value | Notes |
| --- | --- | --- |
| Sample Rate (fs) | 16,000 Hz | LibriSpeech native rate |
| Carrier Frequency (fc) | 4,000 Hz | AM carrier; fc = fs / 4 |
| Pre-Modulation LPF Cutoff | 3,800 Hz | Prevents aliasing at Nyquist |
| Demodulation LPF Cutoff | 3,800 Hz | Removes 2fc artefact |
| Filter Order (Butterworth) | 6 | Steep roll-off, linear phase |
| Modulation Index | 1.000 | 100% AM modulation |
| Noise Type | Pink (1/f) | FFT spectral shaping method |
| SNR Levels (dB) | 25, 18, 12, 8, 5 | 5 channel conditions tested |
| AI Activation Threshold | 5 dB | Smart Routing boundary |
| Whisper Beam Size | 5 | Beam search for ASR |
| Whisper Language | English | Forced-language decoding |
| DNS-64 Input Shape | [1, 1, T] | Batch × Channel × Time |

VI. RESULTS & ANALYSIS

A. Quantitative Results

Table 1 presents the complete WER results for the Baseline (Synchronous Demodulation only), AI-Enhanced (Baseline + DNS-64), and Smart Routing Pipeline across all five SNR conditions. All values represent WER as a percentage; lower values indicate better performance.

Table 1: Word Error Rate (%) Across All System Configurations

| SNR (dB) | Baseline WER (%) | Enhanced WER (%) | Smart WER (%) | Improvement (Smart vs Worst) |
| --- | --- | --- | --- | --- |
| 25 | 5.88 | 23.53 | 5.88 | +17.65 pp |
| 18 | 17.65 | 29.41 | 17.65 | +11.76 pp |
| 12 | 17.65 | 23.53 | 17.65 | +5.88 pp |
| 8 | 52.94 | 117.65 | 52.94 | +64.71 pp |
| 5 | 100.00 | 88.24 | 88.24 | +11.76 pp |
| Average | 38.82 | 56.47 | 36.47 | +18.00 pp |

pp = percentage points. Positive improvement indicates Smart Routing outperforms the worst alternative.

The data reveals three distinct performance zones:

- High SNR Zone (25–12 dB): The Baseline Synchronous Demodulator produces excellent-to-good ASR accuracy (5.88%–17.65% WER). DNS-64 consistently and significantly degrades performance in this zone. Smart Routing correctly bypasses the AI, preserving baseline accuracy.

- Critical Zone (8 dB): Both Baseline and AI paths are severely degraded, but DNS-64 causes catastrophic hallucination (117.65% WER), confirming that the AI is harmful at this SNR. Smart Routing uses the Baseline, reducing WER by 64.71 percentage points over the AI path.

- Emergency Zone (5 dB): The channel is fully collapsed. The Baseline produces 100% WER (no intelligible content). DNS-64 provides a modest recovery (88.24% WER), representing the only scenario where AI assistance is beneficial. Smart Routing correctly activates the denoiser.

B. Transcript Comparison

Table 2 provides a qualitative analysis of the transcription output at each SNR level, allowing direct observation of degradation and recovery patterns.

Table 2: Transcription Output Comparison Across SNR Levels

| SNR | Baseline Transcription | AI-Enhanced Transcription |
| --- | --- | --- |
| Ref. | also a popular contrivance whereby love making may be suspended but not stopped during the picnic season | (ground truth — 17 words) |
| 25 dB | Also a popular condriven, whereby love making may be suspended, but not stopped during the picnic season. | Also a popular contriven, whereby lovemaking may be suspended, but not stopped during the picnic season. |
| 18 dB | Also, a popular contrivence, whereby love-making may be suspended, but not stopped during the picnic season. | Also, a popular contrivance, whereby love-making may be suspended, but does not stop during the picnic season. |
| 12 dB | Also, a popular confidant, whereby love-making may be attended, but you're not stopped, you're like six or seven years old. | Also, a popular confidant, whereby love-making may be suspended, but does not stop in a technical reason. |
| 8 dB | Also, a popular confidant. And by love, maybe they're kind of, but they're not strong. | or for a computer-accommodated man. My last name may be suspended, but that's it. |
| 5 dB | I'm not sure if you may be able to get to the bus stop. | Or is there a space where it can find itself? And I'm not thinking maybe it's intended, but that's enough. |

The transcript data clearly illustrates the AI hallucination effect at 8 dB: while the Baseline produces partially recognisable sentence structures, the AI-enhanced output bears no resemblance to the reference and introduces entirely fabricated content, explaining the 117.65% WER (which can exceed 100% when insertions outnumber reference words).

C. Visual Analysis of Signals

The project generated an extensive suite of high-definition visualisations to support quantitative findings with spectral evidence.

Waveform and Spectrogram Analysis

Log-spectrograms of the original clean speech, the noisy demodulated signal, and the AI-enhanced output at 12 dB reveal the mechanism of DNS-64 damage. The clean spectrogram shows well-defined harmonic structure in the 300–3,500 Hz formant region. The noisy spectrogram shows a uniform noise floor elevated by approximately 15–20 dB. The AI-enhanced spectrogram shows visible spectral holes in the 2,000–3,500 Hz region, corresponding to the fricative consonants (s, t, sh) that DNS-64 incorrectly identified as noise and suppressed, causing substitution errors in the ASR output.

PSD Analysis

Power Spectral Density plots confirm correct AM sideband placement. The modulated signal shows two symmetric sidebands centred at 4,000 Hz, with energy distributed from 200 Hz to 7,800 Hz as designed. The Butterworth LPF frequency response plot confirms a clean passband from 0–3,800 Hz with greater than 60 dB attenuation by 5,500 Hz, validating the anti-aliasing design.

System Stability Heatmap

A transcription fidelity heatmap (100% − WER) across the three pipelines and five SNR levels presents a striking visual summary: the Classical DSP and AI rows each contain red cells (low fidelity) at different SNR points, while the Smart Pipeline row is entirely green-to-yellow, confirming that adaptive routing achieves the highest fidelity at every operating point.

VII. DISCUSSION

A. Interpretation of Results

The most significant finding of this project is that context-unaware AI deployment in a DSP pipeline can be actively harmful. The DNS-64 denoiser produced higher WER than the unaided demodulator at four out of five tested SNR levels. This result is not a failure of the AI model per se — DNS-64 is a state-of-the-art denoiser that performs exceptionally well on its intended domain. Rather, it is a demonstration of the domain mismatch problem: AM channel noise has fundamentally different spectral and temporal statistics compared to the acoustic room noise on which DNS-64 was trained.

The over-processing phenomenon is also clearly evidenced. At 25 dB, the baseline WER is already 5.88% — representing near-perfect transcription of a 17-word sentence with only one erroneous word. Applying DNS-64 increases the WER to 23.53%, as the model attempts to find and remove noise that is virtually absent, inadvertently altering genuine speech features in the process. This is analogous to applying a medical treatment to a healthy patient: the intervention itself becomes the disease.

The transition from Envelope Detection to Synchronous Demodulation was a pivotal engineering decision. The 52.94 percentage point improvement at 12 dB (from 70.59% to 17.65% WER) demonstrates that receiver design is the single most impactful parameter in the system. All other optimisations — AI denoising, beam search parameters, normalisation strategies — produce marginal gains compared to the fundamental choice of demodulation architecture.

The Dynamic SNR Routing approach provides a principled framework for AI-DSP integration. By treating the AI as a conditional resource rather than a universal solution, the Smart Pipeline achieves a 18 percentage point improvement in average WER over the blind AI-enhanced path. This approach mirrors techniques used in industrial noise-cancelling headphones and 5G cellular base stations, where noise reduction algorithms are gated on measured channel quality indicators.

B. Limitations of the Study

Several limitations of the current study should be acknowledged:

- Single utterance evaluation: All results are derived from one 16-kHz utterance from LibriSpeech test-clean. While this enables detailed per-stage analysis, it limits statistical generalisability. A production evaluation would require testing across hundreds of diverse utterances to obtain statistically significant WER estimates.

- Fixed channel model: The pink noise channel model is a simplified approximation. Real AM channels exhibit multipath fading, co-channel interference, intermodulation products, and time-varying SNR conditions that are not captured in this simulation.

- Static SNR routing threshold: The threshold of 5 dB was determined empirically from the limited test data. A more robust system would estimate the SNR in real time from the received signal and adapt the threshold dynamically.

- Domain-mismatched denoiser: DNS-64 was not trained on radio channel noise. The results might improve significantly if a denoiser fine-tuned on AM demodulation artefacts were used, or if the DNS-64 model were re-trained with radio noise augmentation.

- Whisper model size: Development experiments used whisper-small for iteration speed. The large-v3 model would likely produce lower absolute WER values, but the relative ordering between pipeline variants is expected to remain consistent.

VIII. CONCLUSION & FUTURE WORK

A. Conclusion

This project set out to investigate whether AI-based speech enhancement genuinely improves the end-to-end performance of a simulated AM radio communication system. The answer is nuanced and practically important: AI enhancement helps only in extreme channel conditions (SNR ≤ 5 dB) and is actively harmful at all higher signal quality levels.

Three engineering principles emerge from the findings of WhisperComm:

- Receiver design is paramount. The single most impactful improvement across all experiments was the upgrade from Envelope Detection to Synchronous Demodulation. No amount of AI post-processing compensated for the distortion introduced by the wrong demodulation strategy.

- AI is not a universal enhancer. Deep learning models trained on specific noise distributions fail — and fail severely — when presented with out-of-distribution signal degradations. Understanding the training domain of a model is essential before deploying it in a production pipeline.

- Adaptive routing yields the optimal hybrid system. By using measured channel quality as a gating signal for AI activation, the Smart Routing Pipeline achieves the minimum attainable WER at every SNR level, outperforming both the classical DSP and AI-only approaches on average.

The WhisperComm system demonstrates that the most effective AI-enhanced communication systems are those where deep learning augments — rather than replaces — rigorous classical engineering. The integration of both paradigms, with intelligent switching logic, represents the design philosophy best suited to real-world deployment.

B. Future Work

Several avenues present themselves for extending this research:

- Radio-specific fine-tuning: Fine-tune DNS-64 or a similar architecture on a dataset of AM-demodulated noisy speech to address the domain mismatch problem directly. This would likely produce substantial WER improvements in the 5–12 dB SNR range where the AI currently fails.

- Real-time SNR estimation: Implement a real-time SNR estimator (e.g., based on the MMSE log-spectral amplitude estimator) to compute the channel quality dynamically and adjust the routing threshold continuously, rather than relying on a fixed pre-set value.

- Large-scale batch evaluation: Extend the evaluation from a single utterance to the full LibriSpeech test-clean set (2,620 utterances) to obtain statistically valid WER estimates with confidence intervals.

- Frequency-hopping and OFDM channels: Extend the channel model to include OFDM (Orthogonal Frequency Division Multiplexing) or frequency-hopping spread spectrum, which are the dominant physical layer standards in modern digital communications.

- End-to-end differentiable training: Explore training a lightweight neural denoiser end-to-end, using the WER from Whisper as the training loss (via CTC or sequence-level training), to optimise the denoiser specifically for ASR performance rather than general perceptual quality.

- Hardware deployment: Port the Smart Routing Pipeline to an embedded system (e.g., NVIDIA Jetson Nano or Raspberry Pi 5 with Neural Processing Unit) to evaluate real-time latency and power consumption constraints.

IX. REFERENCES

Barker, J., Marxer, R., Vincent, E., & Watanabe, S. (2015). The third CHiME speech separation and recognition challenge: Dataset, task and baselines. 2015 IEEE Workshop on Automatic Speech Recognition and Understanding (ASRU), 504–511.

Boll, S. F. (1979). Suppression of acoustic noise in speech using spectral subtraction. IEEE Transactions on Acoustics, Speech, and Signal Processing, 27(2), 113–120.

Defossez, A., Synnaeve, G., & Adi, Y. (2020). Real time speech enhancement in the waveform domain. Proceedings of Interspeech 2020, 3291–3295.

Ephraim, Y., & Malah, D. (1984). Speech enhancement using a minimum mean-square error short-time spectral amplitude estimator. IEEE Transactions on Acoustics, Speech, and Signal Processing, 32(6), 1109–1121.

Haykin, S. (2001). Communication Systems (4th ed.). John Wiley & Sons.

Kinoshita, K., Ochiai, T., Delcroix, M., & Nakatani, T. (2020). Improving noise robust automatic speech recognition with single-channel time-domain enhancement network. ICASSP 2020, 7009–7013.

Lathi, B. P., & Ding, Z. (2009). Modern Digital and Analog Communication Systems (4th ed.). Oxford University Press.

Lim, J. S., & Oppenheim, A. V. (1979). Enhancement and bandwidth compression of noisy speech. Proceedings of the IEEE, 67(12), 1586–1604.

Panayotov, V., Chen, G., Povey, D., & Khudanpur, S. (2015). LibriSpeech: An ASR corpus based on public domain audio books. ICASSP 2015, 5206–5210.

Proakis, J. G., & Manolakis, D. K. (2007). Digital Signal Processing: Principles, Algorithms, and Applications (4th ed.). Prentice Hall.

Radford, A., Kim, J. W., Xu, T., Brockman, G., McLeavey, C., & Sutskever, I. (2022). Robust speech recognition via large-scale weak supervision. arXiv preprint arXiv:2212.04356.

Vincent, P., Larochelle, H., Bengio, Y., & Manzagol, P. A. (2008). Extracting and composing robust features with denoising autoencoders. Proceedings of ICML 2008, 1096–1103.